Table 1: SWU Credits

| Credit | Sub category | Metric | Pervious Area ( sf) | Impervious Area (sf) | Reduction Percentage | Pervious square foot reduction | Impervious square foot reduction |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Residential | Rain Garden | Storage Volume of Rain Garden | NA |  | Conveyance portion of 3 sf / cubic feet of storage |  |  |
| Residential | Rain Barrel | Storage Volume of Rain Barrel | NA |  | Conveyance portion of 1 sf / 2.5 gallon of storage |  |  |
| Conveyance | Direct Drainage | Lake Mendota, Lake Wingra, Lake Monona, the Yahara River, Wingra (Murphy) Creek, Nine Springs Creek, or Starkweather Creek |  |  | 61.32% |  |  |
| Conveyance | 10-yr No Runoff | Storage volume or Storage plus infiltration if stamped by P.E |  |  | 64.42% |  |  |
| Conveyance | Wetland | Wisconsin Wetland inventory GIS layer from surface water data viewer or Delineated Wetland |  |  | 100.00% |  |  |
| Conveyance | Forest | land is continuous canopy forest |  | N/A | 24.00% |  |  |
| Conveyance | Prairie | land is native prairie not mowed more than 2 times a year |  | N/A | 27.80% |  |  |
| Pollutant Credit |  | 80% TSS reduction |  |  | 35.58% |  |  |
| Rural | Recent Annexations | The property drains to a ditch or ditches, without improved storm sewer facilities; and is not receiving services from City of Madison’s water or sanitary sewer |  |  | 64.42% |  |  |
|  |  |  | Acres | Square Feet |  |  |  |
| Agricultural | Single Parcel | If greater than 5 ac then reduce to 5 ac but keep all impervious |  |  | Reduce to 5 ac total |  |  |
| Agricultural | Multi-Parcel | ii. Parcels that were legally subdivided solely as a result of annexation treated as a single contiguous parcel. |  |  | Reduce to 5 ac total |  |  |